The Endless Square? [SOLUTION]

Code with loop and Wait statements

with Lego;

use Lego;

procedure Turnrover is

Left_Wheel : constant Output_Port := Output_A;

Right_Wheel: constant Output_Port := Output_C;

Left_Rot   : constant Sensor_Port := Sensor_1;

Right_Rot  : constant Sensor_Port := Sensor_3;

begin

Config_Sensor(

Sensor => Left_Rot,

Config => Config_Rotation);

Config_Sensor(

Sensor => Right_Rot,

Config => Config_Rotation);

Output_Power(

Output => Left_Wheel,

Power  => Power_High);

Output_Power(

Output => Right_Wheel,

Power  => Power_High);

Output_Power(

Output => Left_Wheel,

Power  => Power_High);

Clear_Sensor(Left_Rot);

Clear_Sensor(Right_Rot);

Use of enclosing loop (1 pt)

loop

Motors turn in same direction (1 pt)

Output_On_Reverse(Left_Wheel);

Output_On_Reverse(Right_Wheel);

Wait(1000); -- Timing may have to be adjusted

Output_On_Forward(Left_Wheel);

Motors turn in opp. direction (1 pt)

Output_On_Reverse(Right_Wheel);

Wait(500); -- Timing may have to be adjusted

end loop;

end Turnrover;

Code with loops, exit when, and rotation sensors.

There are many ways to implement this, here is one way. Replace the loop in the code above with this loop:

loop

loop

Motors turn in same direction (1 pt)

Output_On_Reverse(Left_Wheel);

Output_On_Reverse(Right_Wheel);

exit when (Get_Sensor_Value(Left_Rot)<-500); -- Timing may have to be adjusted

end loop;

Use of conditional (if or exit statement) (1 pt)

Use of  “Get_Sensor_Value” (1 pt)

Clear_Sensor(Left_Rot);

Clear_Sensor(Right_Rot);

Use of nested loop (1 pt)

loop

Motors turn in opp. direction (1 pt)

Output_On_Forward(Left_Wheel);

Output_On_Reverse(Right_Wheel);

exit when (Get_Sensor_Value(Left_Rot)<300); -- Timing may have to be adjusted

end loop;

Clear_Sensor(Left_Rot);

Clear Sensors (1 pt)

Clear_Sensor(Right_Rot);

end loop;

Pre/Post Conditions, Inputs & Outputs

Precondition: Rover placed on the ground with certain orientation. (1 pt)

Postcondition: Rover will continuously drive in a square like shape.  (1 pt)

Inputs*: Rotation Sensor Values from port 1 and port 2. (1 pt)

Outputs*: Motor Power to ports A and B. (1 pt)

* Since this is a physical system, the inputs and outputs here have been described as measurements from the world. In a more typical program that only runs on a computer, the inputs would be the parameters passed into the procedure and the outputs are the values returned. You will be learning about parameters for procedures and return functions soon.