Driving with Procedures (TEAM) [SOLUTION]

As always, there are many possible variations on the control code. The code below is one possible answer. If you find that you are having difficulty controlling your rover accurately, try the EXACT code below first and tweak it as necessary.

with Lego;

use Lego;

procedure C8 is

Left_Wheel  : constant Output_Port := Output_A;

Right_Wheel : constant Output_Port := Output_C;

Left_Rot    : constant Sensor_Port := Sensor_1;

Right_Rot   : constant Sensor_Port := Sensor_3;

procedure Initialize_Robot is

begin

-- Initialize the Wheels

Output_Power(

Output => Left_Wheel,

Power  => Power_High);

Output_Power(

Output => Right_Wheel,

Power  => Power_High);

-- Initialize the Rotation Sensors

Config_Sensor(

Sensor => Left_Rot,

Config => Config_Rotation);

Config_Sensor(

Sensor => Right_Rot,

Config => Config_Rotation);

end Initialize_Robot;

procedure Go_Forward(Clicks:Integer) is

2 pts – move forward procedure

begin

Output_Power(Left_Wheel,7);

Output_Power(Right_Wheel,7);

Clear_Sensor(Left_Rot);

Clear_Sensor(Right_Rot);

Output_On_Reverse(Left_Wheel);

Output_Reverse(Right_Wheel);

while (((-1*Get_Sensor_Value(Left_Rot))+(-1*Get_Sensor_Value(Right_Rot)))/2<Clicks) loop

if (-1*Get_Sensor_Value(Left_Rot)>-1*Get_Sensor_Value(Right_Rot)) then

Output_Float(Left_Wheel);

Output_On(Right_Wheel);

else

Output_Float(Right_Wheel);

Output_On(Left_Wheel);

end if;

end loop;

Output_Off(Left_Wheel);

Output_Off(Right_Wheel);

end Go_Forward;

2 pts – move backward procedure

procedure Go_Backward(Clicks:Integer) is

begin

Output_Power(Left_Wheel,7);

Output_Power(Right_Wheel,7);

Clear_Sensor(Left_Rot);

Clear_Sensor(Right_Rot);

Output_On_Forward(Left_Wheel);

Output_Forward(Right_Wheel);

while ((Get_Sensor_Value(Left_Rot)+Get_Sensor_Value(Right_Rot))/2<Clicks) loop

if (Get_Sensor_Value(Left_Rot)>Get_Sensor_Value(Right_Rot)) then

Output_Float(Left_Wheel);

Output_On(Right_Wheel);

else

Output_Float(Right_Wheel);

Output_On(Left_Wheel);

end if;

end loop;

Output_Off(Left_Wheel);

Output_Off(Right_Wheel);

end Go_Backward;

1 pt – left turn procedure

procedure Turn_Left(Clicks:Integer) is

begin

Output_Power(Left_Wheel,7);

Output_Power(Right_Wheel,7);

Clear_Sensor(Left_Rot);

Clear_Sensor(Right_Rot);

Output_On_Forward(Left_Wheel);

Output_On_Reverse(Right_Wheel);

while ((Get_Sensor_Value(Left_Rot)+(-1*Get_Sensor_Value(Right_Rot)))<Clicks) loop

if (Get_Sensor_Value(Left_Rot)>-1*Get_Sensor_Value(Right_Rot)) then

Output_Float(Left_Wheel);

Output_On(Right_Wheel);

else

Output_Float(Right_Wheel);

Output_On(Left_Wheel);

end if;

end loop;

Output_Off(Left_Wheel);

Output_Off(Right_Wheel);

end Turn_Left;

procedure Turn_Left_90 is

begin

Turn_Left(350); -- YOU MIGHT WANT TO TWEAK THIS NUMBER

end Turn_Left_90;

procedure Turn_Right(Clicks:Integer) is

1 pt – right turn procedure

begin

Output_Power(Left_Wheel,7);

Output_Power(Right_Wheel,7);

Clear_Sensor(Left_Rot);

Clear_Sensor(Right_Rot);

Output_On_Reverse(Left_Wheel);

Output_On_Forward(Right_Wheel);

while ((-1*Get_Sensor_Value(Left_Rot)+Get_Sensor_Value(Right_Rot))<Clicks) loop

if (-1*Get_Sensor_Value(Left_Rot)>Get_Sensor_Value(Right_Rot)) then

Output_Float(Left_Wheel);

Output_On(Right_Wheel);

else

Output_Float(Right_Wheel);

Output_On(Left_Wheel);

end if;

end loop;

Output_Off(Left_Wheel);

Output_Off(Right_Wheel);

end Turn_Right;

procedure Turn_Right_90 is

begin

Turn_Right(350); -- YOU MIGHT WANT TO TWEAK THIS NUMBER

end Turn_Right_90;

begin

-- THE WAVY LINE

Initialize_Robot;

loop

Go_Forward(200);

Turn_Right_90;

Go_Forward(200);

Turn_Left_90;

Go_Forward(200);

Turn_Right_90;

Go_Forward(200);

Turn_Left_90;

Go_Forward(200);

Go_Backward(200);

Turn_Right_90;

Go_Backward(200);

Turn_Left_90;

Go_Backward(200);

Turn_Right_90;

Go_Backward(200);

Turn_Left_90;

Go_Backward(200);

end loop;

end;